--------------------------------------------------------------------------------

--[VBE32.E] v1.0 DOS Interrput functions for the Euphoria EX.EXE Interpretor	--

--(C)2004. Hayden McKay, All rights reserved. Public Domain.                  --

--------------------------------------------------------------------------------

*** DISCLAIMER ***

------------------------------------------------------------

-- I accept no responsibilty for any missuse, dammage,    --

-- fraud or any other event that may arise from the use   --

-- of this software.                                      --

------------------------------------------------------------

--------------------------------------------------------------------------------

COMMENTS:	This libraries purpouse is to provide Euphoria DOS32 coders with a

basic set of VGA BOIS interrupt routines. What you do with theese

routines is up to you. You may want to create a structure that holds

vendor information etc...

It is wise to get a copy of the 'VBE Standard Core Functions' wich

is freely available for download at www.vesa.org and read it though

before attempting to use any of these functions. There is alot of

things to consider before calling some of these functions, so by

reading through the 'VBE Standard Core Functions' you will get a

better understanding on what these functions do, what they are

used for and what how to use them.

I am currently working on a VBE30.E module wich will be a complete

protected mode graphics engine for the Euphoria DOS interpretor.

It will have a deifferent set of routines. You will no longer have

to call any of the VBE32.E directly anymore. The VBE30.E module will

completely and independantly handle Video mode setup and provide a

set of routines for accelerator, sterioscopic functions and more...

Bareing this in mind you should not count on VBE32.E to be any more

that what you currently see.

--------------------------------------------------------------------------------

-- global function VBE_Return_Status() -----------------------------------------

i = VBE_Return_Status()

Returns the last error status.

n.b. All functions return some kind of error status when called. This function

really only needed be used in the case where you are not cheaking the return

values from calls made by functions this library. eg: you may only want to check

for an error at a specific point in your code or you may want to retieve the

last error after sebsequent calls.

'i' will be the error status.

Error codes have been defined as follows;

SUCCESS		=	#004F		Call was successful.

FAIL			=	#014F		Call failed.

NOT_SUPPORTED 	= 	#024F		Not supported with current hardware config.

INVALID		=	#034F		Invalid in current video mode.

--------------------------------------------------------------------------------

-- global function VBE_Controller_Information() --------------------------------

i = VBE_Controller_Information()

Retrieves the VGA BOIS controller information.

The information is placed in a global sequence labeled as 'VbeInfoBlock'

For information on the block structure you will need to consult www.vesa.org

for a copy of the 'VBE Standard Core Functions' wich is freely available for

download.

'i' will be VBE_Return_Status()

--------------------------------------------------------------------------------

-- global function VBE_Mode_Information(integer Mode) --------------------------

i1 = VBE_Mode_Information(i2)

Retrieves a specific mode information.

'i2' is the video mode for wich you desire the information.

The information is placed in a global sequence labeled as 'ModeInfoBlock'

For information on the block structure you will need to consult www.vesa.org

for a copy of the 'VBE Standard Core Functions' wich is freely available for

download.

'i1' will be VBE_Return_Status()

NOTE: Not all video cards support this one.

--------------------------------------------------------------------------------

-- Set_VBE_Mode(integer Mode) --------------------------------------------------

x = Set_VBE_Mode(i)

Sets a desiered video mode.

'i' is the desired video mode to set.

On success 'x' will be a two element sequence;

x[1] will be SUCCESS, x[2] will be the mode set.

On fail 'x' will be an integer, a return error.

--------------------------------------------------------------------------------

-- global function Current_VBE_Mode() ------------------------------------------

x = Current_VBE_Mode()

Retrieves the current video mode.

On success 'x' will be a two element sequence;

x[1] will be SUCCESS, x[2] will be the mode set.

On fail 'x' will be an integer, a return error.

--------------------------------------------------------------------------------

IMPORTANT: 	I have had some problems with this one so I commented it out.

Uncomment and use at your own risk!

global function Save_Restore_State(integer Action, integer Request, atom Buffer)

x = Save_Restore_State(i1, i2, a)

This function can do one of three actions;

1. ReturnState			Return the buffer size.

2. SaveState			Save a specific state.

3. RestorState			Restore a specific state.

The Action can be performed on one of four requests;

1. Hardware

2. BIOS

3. DAC

4. Register

'i1' is the action to perform.

'i2' is what to permorm the action on.

'a' is a flat address to a previously allocated memory block.

n.b. 'a' is only used for ReturnSate and is ignored by other actions.

On success 'x' will be a two element sequence;

x[1] will be SUCCESS.

x[2] will be number of 64k blocks to hold the state buffer. (ReturnSate only)

On fail 'x' will be an integer, a return error.

--------------------------------------------------------------------------------

-- global function Display_Window_Control(integer Action, atom Units) ----------

x = Display_Window_Control(i, a)

This function can do one of four actions;

1. SetMemoryWindowA		Set memory page 0.

2. GetMemoryWindowA		Get memory page 0.

3. SetMemoryWindowB		Set memory page 1.

4. GetMemoryWindowB		Get memmpy page 1.

'i' is the the desired action to perform.

'a' is the window number in granualty units.

On success 'x' will be a two element sequence;

x[1] will be SUCCESS.

x[2] will be the window number in granualty units.

On fail 'x' will be an integer, a return error.

--------------------------------------------------------------------------------

-- function SetGet_Scan_Line(integer Action, atom Width) -----------------------

x = SetGet_Scan_Line(i, a)

This function can do one of four actions;

1. SetScanLinePixels		Set scanline in pixels.

2. GetScanLine			Retrive the scanline information.

3. SetScanLineBytes		Set scanline in bytes.

4. GetMaximumScanLine		Get the maximum scanline values.

'a' is the desired width. n.b. Not used for GetMaximumScanLine and GetScanLine.

On success 'x' will be a three element sequence;

x[1] will be bytes per scanline.

x[2] will be maximum number of scanlines.

x[3] wil be actual pixels per scanline.

On fail 'x' will be an integer, a return error.

-------------------------------------------------------------------------------

- global function SetGet_Display_Start(integer Action, atom Pixel, atom Scan) -

x = SetGet_Display_Start(i1, i2, i3)

This function can do one of three actions;

1. SetDisplayStart			Set the display start.

2. GetDisplayStart			Get the display start.

3. SetDisplayStartVertical		Set the display start during vertacle ret.

'i1' is the action to perform.

'i2' is the first displayed pixel in the scanline.

'i3' is the first displayed scanline.

On success 'x' will be a three element sequence;

x[1] is reserved and will be NUL(0).

x[2] will be first displayed scanline.

x[3] wil be first displayed pixel.

On fail 'x' will be an integer, a return error.

-------------------------------------------------------------------------------

-- global function SetGet_DAC_Palette(integer Action, integer Bits) -----------

x = SetGet_DAC_Palette(i1, i2)

This function can do one of two actions;

1. SetDACPalette		Set DAC Palette Format.

2. GetDACPalette		Get DAC Palette Format.

'i1' is the action to perform.

'i2' is the desired number of bits. n.b. ignored on GetDACPalette

On success 'x' will be a two element sequence;

x[1] will be SUCCESS, x[2] will be the current number of DAC bits.

On fail 'x' will be an integer, a return error.

n.b. Remember to reset the DAC bits back to 6 whan you finished if you change

them.

-------------------------------------------------------------------------------

-- global function SetGet_Palette_Data(integer Function, integer Palette, -----

------------------------------------------------ integer First, atom Address) -

i = SetGet_Palette_Data(i1, i2, i3, a)

This function can do one of five actions;

1. SetPaletteData			Sets the palette data.

2. GetPaletteData			Gets the palette data.

3. Set2ndPaletteData		Sets the 2nd palette data.

4. Get2ndPaletteData		Gets the 2nd palette data.

5. SetPalettDataDuringVR	Sets pallete data during a verticle retrace.

'i1' is the action to perform.

'i2' is the number of palette regiters to update.

'i3' the first palette register to update.

'a' pointer to a buffer that contains a table of palette registers.

Palette table format:     {Alignment, Red, Green, Blue}

'i' wil be VBE_Return_Status()

-------------------------------------------------------------------------------

-- global function Get_Protected_Mode_Interface() -----------------------------

x = Get_Protected_Mode_Interface()

On success 'x' will be a two element sequence.

x[1] will be a segment:offset style pointer.

x[2] will be the length of the table in bytes.

On fail 'x' will be an integer, a return error.

For information on the block structure you will need to consult www.vesa.org

for a copy of the 'VBE Standard Core Functions' wich is freely available for

download.

-------------------------------------------------------------------------------